Monthly Progress Report of Exploratory Project

Name of the Students			Roll No				Signature

1.   Ishant Singh                                          24045059

2.   Mohit Jain                                             24045079

3.   Ranit Mondal                                        24045105

Title of the Project: Review on Covalent Organic Frameworks (COF) for Co2 Capture and Industrial Applications

Progress of Month:  September / October / November/ December (Tick )

Progress Report (Maximum 100 words):

Literary Review on Covalent Organic Frameworks for CO2 Combustion and Industrial Application:

- Porosity and High Crystalinity
COFs (Covalent Organic Frameworks) offer good pore sizes and high surface areas due to their crystalline and modular structure. This tunability enables selective CO₂ capture by designing pore environments that favour CO₂ over other gases like N₂ or CH₄.

- Impact of 2D vs. 3D Structures
2D COFs form layered sheets with π–π stacking, ideal for gas separation due to narrow, ordered channels. 3D COFs provide interconnected pore networks with higher surface areas and enhanced mechanical stability, making them suitable for industrial environments.

- Effective Synthesis Methods
New synthesis techniques using heteropoly acid catalysts and room-temperature processing have reduced reaction times (from 72 to 2 hours) and energy consumption. These methods lower production costs by 40–50%, making COFs scalable and commercially viable.

- Functionalization for Enhanced Performance
Functionalizing COFs with amines or ionic liquids significantly increases CO₂ affinity and selectivity. Materials like PPN6DETA and AelmBr-TAPT show up to 15× improved uptake compared to non-functionalized COFs.

- Layer Stacking and Pore Optimization
Controlled stacking (eclipsed, staggered, inclined) and fine-tuned pore sizes (~1.0 nm) enhance CO₂/N₂ separation by creating efficient adsorption pathways and maximizing CO₂ interaction with functional sites.

- Superior to Conventional Technologies
Compared to MEA-based systems and solid amines, COFs offer 30–40% lower operating energy requirements and cost without corrosion issues or high thermal degradation.

- Excellent Thermal and Mechanical Stability
COFs exhibit strong durability, with some variants maintaining structure up to 500–600°C and withstanding over 10,000 pressure cycles. This robustness is crucial for long-term industrial deployment.

Grade provided by the supervisor: A           / B             /     C                (Tick)

Signature of the Supervisor with date:

Note:   1. Back side of this page can be used, if needed, for diagram, photograph related with the project.

2.  More than one page will not be accepted.

3.  For any query related with the project contact your TA:

Mr. Kushagra Singh– kushagrasingh.che24@itbhu.ac.in